Overview of Latches and Flip Flops

John Renner

- SR Latch

The SR latch is built by cross-coupling two NAND or NOR gates together. It has two inputs (S and R) and two outputs (Q and Q’). Outputs Q and Q’ are inverse.  If Q=0, then asserting S causes Q to become 1. But if Q is already 1, then asserting S has no change on the output. If Q=1, then asserting R causes Q to become 0. If Q is already 0, then asserting R has no change on the output. If neither S nor R are asserted, then the output of the latch will hold its value. Asserting both S and R at the same time is invalid, and causes undefined behavior.

- D Latch

A D Latch is created by adding a NOT gate to the R input of an SR Latch. The input of this NOT gate is then fed directly from the S input. Therefore, the D Latch only has one input. When the input is high, the S input of the latch is set. Once the input transitions to an off state, the R input of the SR latch is set. Because of this, the output of the D latch simply follows the input. However, this eliminates the illegal (S=1, R=1) state.

- Gated Latch

A Gated Latch can be created by ANDing the S and R inputs of an SR Latch with a third input, typically a clock. Whatever changes occur to the S and R inputs can only propogate through these new AND gates to the latch when the third signal is also ON. The third signal acts as a gate that enables or disables the inputs to the latch.

SR Latches, D Latches, and JK Latches can all be designed with gated inputs.

- JK Latch

The JK Latch is an improvement on the SR Latch that uses feedback to eliminate the undefined behavior of the (S=1, R=1) state. The S and R inputs of the SR latch are fed by inputs J and K, which are ANDed with Q’ and Q, respectively. Therefore, when Q=0, the gate that feeds the R input is effectively “off”, and the gate feeding the S input is “on”. When Q=1, the opposite is true. Because only one input gate is enabled at a time, then J and K can both be asserted simultaneously. The result of this would be a transition of the output state, regardless of its current state.

- SR Flip Flop

Latches are triggered solely by their inputs, which can arrive at any time. This makes them asynchronous. Flip Flops, on the other hand, are synchronized to a clock signal, and the outputs only change during the rising or falling edge of the clock cycle.

The SR Flip Flop consists of two SR Latches L1 and L2 back to back (master-slave), gated with a clock that sends an inverted clock signal to the L1. When the clock is off, L1 is transparent, and follows the present input. L2 holds its current state. When the clock signal goes high, L2 becomes transparent, and any change at the input of the Flip Flop propogates through to the output. L1 then turns off, preventing any further changes at the input from reachng L2.

While the SR Flip Flop appears to only trigger on the rising edge of the clock, it is possible for the output to respond to inputs that do not occur on the rising edge. For example, if Q=0 and S is active during the falling clock edge but inactive before the rising clock edge, then Q will become 1, even though no input was applied at the moment of the active edge.

- JK Flip Flop

The JK Flip Flop has a similar master-slave configuration as the SR Flip Flop, but uses JK latches instead of SR latches. It eliminates the invalid state of the SR Flip Flop by using feedback from the outputs. Like other flip flops, it changes state only on a clock edge (rising or falling, depending on design). When J=0 and K=0, the output holds its current value. When J=1 and K=0, the output is set to 1. When J=0 and K=1, the output is reset to 0. When J=1 and K=1, the output toggles to the opposite of its current state.

This makes the JK Flip Flop more versatile than the SR Flip Flop, since all input combinations are valid and predictable. However, JK Flip Flops still suffer from the same transparency window issue as SR Flip Flops, where inputs applied on the clock’s falling edge can produce changes to the output on the next rising edge.

- D Flip Flop

The D Flip Flop is a master-slave configuration of two D latches. The master latch is enabled during one phase of the clock, and the slave latch is enabled during the opposite phase. Like the SR and JK Flip Flops, the output updates only on the active clock edge when the slave latch captures the value stored in the master.

Unlike the SR Flip Flop, the D Flip Flop has a single data input and no invalid input combinations. Its next state is determined solely by the value of D at the clock edge. Since the reset input is driven by the complement of D, removing a high on D immediately resets the latch, preventing short pulses from being latched as they can be in an SR or JK Flip Flops.

- T Flip Flop

The T (Toggle) Flip Flop is a clocked storage element whose output changes state only when its input T is asserted. It can be implemented using a master–slave configuration of either JK or D latches. In a JK implementation, the J and K inputs are tied together to form the single T input. In a D implementation, the input is formed by feeding the output back into an XOR gate, causing the next state to depend on the current output.

When T = 0, the flip flop holds its current state at the active clock edge. When T = 1, the output toggles to the opposite of its present state at each active clock edge.